# ANNEX II + III:	 TECHNICAL SPECIFICATIONS + TECHNICAL OFFER

# Contract title: Supply of Equipment	p 1 /…

Publication reference: BGTR0200074-PP2-SUP-01

Columns 1-2 should be completed by the contracting authority

Columns 3-4 should be completed by the tenderer

Column 5 is reserved for the evaluation committee

Annex III - the contractor's technical offer

The tenderers are requested to complete the template on the next pages:

- Column 2 is completed by the contracting authority shows the required specifications (not to be modified by the tenderer),

- Column 3 is to be filled in by the tenderer and must detail what is offered (for example the words ‘compliant’ or ‘yes’ are not sufficient)

- Column 4 allows the tenderer to make comments on its proposed supply and to make eventual references to the documentation

The eventual documentation supplied should clearly indicate (highlight, mark) the models offered and the options included, if any, so that the evaluators can see the exact configuration. Offers that do not permit to identify precisely the models and the specifications may be rejected by the evaluation committee.

The offer must be clear enough to allow the evaluators to make an easy comparison between the requested specifications and the offered specifications.

All power cables must comply with TSE and ISO standards.

| 1. Item number | 2. Specifications required | 3. Specifications offered | 4. Notes, remarks, ref to documentation | 5. Evaluation committee’s notes |
| --- | --- | --- | --- | --- |
| 1 | Conference Chair – 116 pieces Steel frame, firm foam upholstery, armrests, fixed to the floor, automatic folding seat. Seat dimensions: width approx. 50 cm-55 cm; depth 40 cm~45 cm, back height of 85 cm~90 cm |  |  |  |
| 2 | Podium Stage – 14 m2 1)Raw MDF – 14 m2 stage platform box profiles, 25 mm thick E1 class MDF to be mounted onto box profiles with countersunk screws at 25 cm spacing 2)LAMINATE FLOORING – 14 m2: Profiles covered with 8 mm thick, 31 class laminate flooring. Underlay to be laid and installed as floating system on stage and steps |  |  |  |
| 3 | Floor Tiles and Carpets – 140 m2 Made from 100% Polyamide 6 stain-resistant material, Wall-to-wall contract carpet Usage: Heavy traffic Production: Chromojet printing Total thickness: 8.2 mm Pile weight: 500 g/m² Total weight: 1490 g/m² Base: woven polypropylene + actionback/feltback Classification: Usage class 33, comfort class LC2 Fire rating: Cfl-s1 |  |  |  |
| 4 | Wall Acoustic Panel -144 m2 18mm MDFLAM DOUBLE-SIDED MILLED ACOUSTIC WALL PANEL According to the project and detailed drawings, acoustic MDFLAM panels will be placed on existing surfaces, aligned with reflective MDFLAM wall cladding and spaced approx. every 50.0 cm.(Distance from existing wall: 40.0 mm–160.0 mm as required by acoustic values.) Total thickness: 17.0 mm–20.0 mm Core: E1 class MDF, min. 16.0 mm–18.0 mm Front surface: laminated Back surface: fabric or acoustic textile .Front slot dimensions: 4 mm groove width, 8 mm depth, 30mm spacing.Rear slot dimensions: 4 mm width, 13mm depth, 30 mm spacing Panel length: approx. 140–280 cm.Panel width: approx. 50–105 cm (adjustable for acoustic/special needs) .Edges: tongue-and-groove or 90° square.Texture, pattern, quality must match reflective MDFLAM panels.Installation must be level and aligned with reflectiveMDFLAM panels.Panels fixed using adhesives, screws, concealed fasteners.No fixing element shall be visible on the finished wooden cladding. 18mm MDFLAM REFLECTIVE WALL PANEL According to the project and detailed drawings, wall panels made of MDFLAM will be installed on existing building surfaces. The dimensions will be adjusted according to the wall cladding production, and panels will be placed approximately every 50.0 cm.(The distance between panels and the existing structure will be between 40.0 mm and 160.0 mm in accordance with acoustic values.)Panels will have a total thickness of 17.0 mm–20.0 mm, made of E1 class MDF with 16.0 mm–18.0 mm thick MDF core; front surfaces will have laminated finish; edges preferably 90° square (or beveled / tongue-and-groove as required).The texture, pattern and quality will match double-sided milled MDFLAM acoustic panels as much as possible.Panels will be installed horizontally and vertically leveled and aligned with double-sided milled MDFLAM panels with no misalignment.They will be fixed using special adhesives, nails, screws, concealed wooden wedges, connection parts or similar fasteners.No fixing element (nail, screw, dowel, etc.) shall be visible on the finished surface. |  |  |  |
| 5 | Acoustic Baffle Ceiling - 114 m2 12mm CEILING FELT ACOUSTIC PANEL Raw material: 100% Polyester Fiber (100% recycled) Weight and Thickness: 3040 g/m²; 12 mm Color: FD Color Chart Length: 300 cm Width: Custom dimensions Thickness: 12 mm NRC(ASK12-R2 series): 0.25-0.55 Certification: Reach / RoHS / Oeko-tex / TSE / ISO 9001 Fire Class: BS1d0 Health Benefits: 99.9% free of harmful substances, antibacterial, anti-carcinogenic. Thermal Insulation: 0.034 λ W/mk Water Vapor Diffusion: 1.1 μ |  |  |  |
| 6 | Ceiling + Floor Skirting Board Set - 9108 cm. 10 cm HEIGHT MDFLAM BASEBOARD E1 class MDF, min. 16–18 mm thickness, laminated finish, edges 90°. Applied with silicone adhesive at wall-floor junctions; gaps filled with transparent silicone. Floor skirting: 4554 cm Ceiling skirting: 4554 cm |  |  |  |
| 7 | Steel & Iron Construction For Wall And Floor – 144+140 = 284 m2 Wall studs: 40x80x2 mm box profile at 60 cm center spacing Intermediate connectors: 40x40x2 mm box profile at 120 cm spacing Floor main beams: 40x80x2 mm box profile at 100 cm spacing Floor connectors: 40x40x2 mm box profile at 50 cm spacing All profiles will be factory-painted. |  |  |  |
| 8 | Acoustic Entrance Door (Double Wing) – 1 piece Each wing is made of MDF laminate material with dimensions of 90x210 |  |  |  |
| 9 | Logo – 1 piece 150 x 150 cm in size, made from 3 mm laser-cut plexiglass |  |  |  |
| 10 | Surface-mounted linear lighting. – 46 pieces Aluminum casing, electrostatic paint, LED. 4000K Natural white, 220-240 V AC / 50-60 Hz 50 x 80 x 1200 mm ELECTRICAL WORKS for 46 units of surface-mounted linear lighting: All electrical cables must comply with TSE & ISO standards. |  |  |  |
| 11 | Digital Platform – 1 piece 22" Full HD Multi Touch Capacitive Touch Screen intel i3 GHz 4GB Ram, 120 GB SSD, Win 11 Pro 3x2 switcher with 2x HDMI and 1x Type-C input 1 pcs 18" Gooseneck Condenser Microphone – 1 pcs 150m HD, 120m 4K60Hz, HDBase-T Extender, Audio out – 1 pcs |  |  |  |
| 12 | Professional Sound System |  |  |  |
| 12.1 | 11 Mono / 2 Stereo or 8 mono / 4 stereo 4 Aux - 8 Ch. Compr. - Phantom Power +48V Spx 24 Program Digital Effect - 19'' Rack Mountable USB - 1 piece |  |  |  |
| 12.2. | 2 Ch Amplifier. 2x400 Watt 4 Ohm, 1x800 Watt bridge 8 Ohm, XLR In/Out, Spekon Out, Limiter - 2 pieces |  |  |  |
| 12.3. | 2-way Passive speaker - 8" + 1" – 160 Wrms – 6 pieces |  |  |  |
| 12.4. | U bracket horizontal wall mount J 8 – 6 pieces |  |  |  |
| 12.5. | Single Lavalier Wireless System with CVL Lavalier Microphone – 2 pieces |  |  |  |
| 12.6. | Single Handheld Wireless System with PG 58 Microphone - 2 pieces |  |  |  |
| 12.7. | 19" Rack Cabin and Accessories- 1 piece |  |  |  |
| 12.8. | Delegate Central Unit – 1 piece |  |  |  |
| 12.9. | Delegate Microphone – 4 pieces |  |  |  |
| 12.10. | 20 m cable -1 piece |  |  |  |
| 13 | Professional Video System |  |  |  |
| 13.1. | Indoor LED Screen / P1,86 3200 mm x 1760 mm 5,63 m 23840 Hz >600 nit brightness Construction Included – 1piece |  |  |  |
| 13.2. | 4×1 HDMI 4K UHD Switcher 18Gbps – 1 piece |  |  |  |
| 14 | Professional Lighting System |  |  |  |
| 14.1. | RGBWAUV 6in1 LED WASH LIGHT DMX512 & Automatic & Remote Control Led: 18 x 12W Red-Green-Blue-White Amber-Ultraviolet 6/10 Ch. DMX, 2.5 Kg – 4 pieces |  |  |  |
| 14.2. | 192 CH. DMX 16CH. MOVING HEAD CONTROL DESK 12 SCANNER/16CH. 30 BANKS 6 CHASES 240 SCENES - 1 piece |  |  |  |
| 14.3. | Light Hanging Fixture – 4 pieces |  |  |  |
| 14.4. | Safety Rope 1 m (Silver) – 4 pieces |  |  |  |
| 15 | Video Conference System |  |  |  |
| 15.1. | AI Auto Tracking PTZ Cameras – 1 piece |  |  |  |
| 15.2. | Sound Card – 1 piece |  |  |  |
| 15.3. | USB 2.0 480 Mbps Cat6 Extender, 100 m – 1 piece |  |  |  |
| 16 | Cables |  |  |  |
| 16.1. | 2x0,22 mm Black Microphone Cable 1 Roll 100 m – 4 pieces |  |  |  |
| 16.2. | 2x1,5 mm² Dark Gray Speaker Cable 1 Roll 100 m – 2 pieces |  |  |  |
| 17 | Connectors Interconnectors - 1 piece |  |  |  |
| 18 | Services Audio installation, commissioning, engineering, training (Kirklareli) – 1 piece |  |  |  |